Xn Timers:

TXnRELOCprep:

- Phase: Preparation

- Start Trigger: Upon transmission of the HANDOVER REQUEST message from the Source gNB to the Target gNB.

- Stop Trigger: Upon reception of the HANDOVER REQUEST ACKNOWLEDGE message from the Target gNB.

- Expiry Action: The Source gNB aborts the handover procedure. The UE remains connected to the Source cell.

TXnRELOCoverall

- Phase: Execution & Completion

- Start Trigger: Upon transmission of the Handover Command (RRCReconfiguration) to the UE.

- Stop Trigger: Upon reception of the UE CONTEXT RELEASE message from the Target gNB.

- Expiry Action: The Source gNB forcefully releases the UE resources and deletes the UE context.

N2/NG Timers: Specifically for NG-Based Handover

TNGRELOCprep

- Phase: Preparation

- Start Trigger: Upon transmission of the HANDOVER REQUIRED message from the Source gNB to the AMF (5G Core).

- Stop Trigger: Upon reception of the HANDOVER COMMAND message from the AMF.

- Expiry Action: The Source gNB cancels the handover.

TNGRELOCoverall:

- Phase: Execution & Completion

- Start Trigger: Upon transmission of the Handover Command (RRCReconfiguration) to the UE.

- Stop Trigger: Upon reception of the UE CONTEXT RELEASE COMMAND from the AMF.

- Expiry Action: The Source gNB releases all resources related to the UE context.

RRC Timers:

T301:

- Start Trigger: Upon transmission of the RRCReestablishmentRequest message.

- Context: This happens immediately after a Handover Failure (e.g., T304 expired) or a Radio Link Failure (RLF).

- Stop Trigger: Upon reception of an RRCReestablishment or RRCSetup message, or if the selected cell becomes unsuitable.

- Expiry Action: The UE must transition to RRC_IDLE.

- Impact: All active data sessions are dropped; the UE must start from scratch to find the network.

T304:

- Phase: Execution

- Start Trigger: There are three main triggers relevant to the NTN scenario:

  - Standard HO: Reception of RRCReconfiguration including reconfigurationWithSync.

  - Conditional HO (CHO): Upon execution of a stored CHO configuration (i.e., when the time/location condition is met).

  - LTM (L1/L2 Triggered Mobility): Upon indication from lower layers that a cell switch is triggered (a high-speed Rel-18 feature).

- Stop Trigger:

  - Successful completion of Random Access (RACH) on the target cell (SpCell).

  - Indication of successful RACH-less handover completion from lower layers.

- Expiry Action: Initiate RRC Re-establishment procedure: The UE declares a Handover Failure (HOF), stops trying to connect to the target, and starts searching for any available cell to recover the connection (triggering T311).

T310:

- Phase: Pre-Handover / Monitoring

- Start Trigger: Upon detecting physical layer problems for the SpCell (receiving N310 consecutive out-of-sync indications from lower layers). CSV Logic: If the PRX stays below a certain threshold for N310 steps, this timer starts.

- Stop Trigger:

  - Receiving N311 consecutive in-sync indications (Signal recovered).

  - Receiving an RRCReconfiguration with reconfigurationWithSync (The Handover Command arrived just in time!).

  - Initiating the connection re-establishment procedure manually.

- Expiry Action:

  - If security is not active: Go to RRC_IDLE.

  - If security is active: Initiate the RRC connection re-establishment procedure (Transitions to T311).

- Explanation of N310, N311: A threshold constant. It uses an event-counter.

  - N310: The threshold for declaring a "Physical Layer Problem." (Low-quality signal)

    - The Event being counted: Out-of-Sync (OOS) Indications.

    - Trigger: Sent by the UE hardware (Layer 1) when it cannot reliably decode the satellite's command channel (PDCCH) because the error rate (BLER - Block error rate) is above 10% (Qout).

    - Mechanism:

      - The UE monitors the satellite signal.

      - Each time it fails to "hear" a radio frame, the N310 counter increments by 1.

      - If the signal is good for even one frame, the counter resets to zero.

      - If the counter reaches the value of the Constant N310 (e.g., 10), the UE starts Timer T310.

  - N311: The threshold for declaring "Link Recovery."

    - The Event being counted: In-Sync Indications.

    - Trigger: Sent by the UE hardware when it can decode the satellite signal very clearly, with an error rate below 2% (Qin).

    - Mechanism:

      - While the failure timer (T310) is running, the UE looks for a "comeback" signal.

      - Each time it successfully decodes a frame, the N311 counter increments by 1.

      - If the counter reaches the value of the Constant N311 (e.g., 5), the UE stops and resets Timer T310.

T311:

- Phase: Recovery (Post-Failure)

- Start Trigger: Upon initiating the RRC connection re-establishment procedure (triggered by T304 expiry, T310 expiry, or T312 expiry).

- Stop Trigger: Upon selection of a suitable NR cell (finding a new satellite beam/gNB to talk to).

- Expiry Action: The UE enters RRC_IDLE. All data sessions are dropped.

T312:

- Phase: Pre-Handover / Monitoring

- Start Trigger: Triggered when the UE sends a Measurement Report while the T310 (Radio Link Failure) timer is already running.

- Stop Trigger:

  - Receiving N311 (In-sync indications).

  - Receiving RRCReconfiguration with reconfigurationWithSync (The Handover Command arrived!).

  - Expiry of T310.

- Expiry Action: Initiate the RRC connection re-establishment procedure.

T430:

- Phase: Preparation / Execution (NTN Specific)

- Start Trigger:

  - Receiving SIB19 (Satellite orbit/speed data).

  - Receiving an RRCReconfiguration with reconfigurationWithSync.

  - Performing a Satellite Switch with resynchronization.

- Stop Trigger: Receiving a new RRCReconfiguration or another Satellite Switch.

- Expiry Action (per Section 5.2.2.6):

  - The UE must mark all stored satellite position/timing data as invalid.

  - The UE is forbidden from transmitting (it cannot RACH or send data).

  - The UE must immediately re-acquire SIB19 to get fresh coordinates.

| Timer | Duration | Mục đích |
| --- | --- | --- |
| T301 | 2s | RRC re-establishment |
| T304 | 100ms | Handover execution |
| T310 | 1s | Radio link monitoring |
| T311 | 3s | Cell selection |
| T312 | 500ms | Measurement reporting |
| T430 | 30s | NTN satellite info validity |
| TXnRELOCprep | 3s | Xn handover preparation |
| TXnRELOCoverall | 6s | Xn handover execution |
| TNGRELOCprep | 5s | N2 handover preparation |
| TNGRELOCoverall | 10s | N2 handover execution |